# JS Air Curtains Dam, Air Curtain Specification

- The Dam Air Curtain unit features a modern, clean design for vertical or horizontal installation, with self-supporting galvanised steel casing.

- The Dam casing is finished with a structural epoxy-polyester paint finish, offered in standard RAL 9016 with other RAL classic colours, or stainless steel, available on request. The front face can also be customised with logos, lighting, signage, or lettering to suit branding or design needs.

- As standard, the Dam fans are ‘AC’ type, with double inlet centrifugal design, featuring low noise levels. They use an external rotor motor, offer five selectable speeds, and come equipped with built-in thermal protection. Two power options are available to accommodate various mounting heights and operating conditions.

- EC (electronically commutated) fans with low energy requirements can be specially ordered, offering enhanced energy efficiency, lower operating costs, and sophisticated control options.

- The Dam unit shall measure 260mm high by 580mm deep and can be supplied in various lengths ranging from 1.0m to 3.0m in 0.5m increments.

- The unit is suitable as standard for exposed, horizontal mounting above door, hung using M8 threaded drop rods. An optional base plate with floor fixing points shall be available, to allow vertical mounting to side of the doorway. Optional brackets may be specially ordered for overhead or wall mounting, including, wall ‘rail’ supports and suspension cables.

- An optional Dam Twin system is available, comprising two vertically opposed Dam air curtains with reversed airflow. Suited to adverse environments, this double jet system works as a closed circuit creating a separation zone at the door entrance.

- A clean front panel replaces the traditional grille on the Dam unit with double air inlet areas located behind the front panel, so only routine cleaning or vacuuming is needed in normal conditions.

- The unit can be specified with various regulated heating options: ‘A’ type (low-energy air only, unheated or recirculating ambient air), ‘E’ type (electric element heating with three stages), or ‘P’ type (LPHW water coil).

- E’ type electric units shall operate on 400V/50Hz three-phase power, include power contactors and feature shielded elements, with three standard heating stages.

- ‘P’ type water heated units shall offer on/off or proportional heating control, depending on valve type, with coil options for 50°C, 60°C or 80°C flow. External solenoid valves shall also be available on request.

- The unit comes with anodised aluminium vanes shaped like air foils to minimise drag and noise. Each vane can be adjusted from 0 to 15º from the vertical to control the direction of airflow.

- The Dam is equipped as standard with an ‘Advanced Pro’ digital controller, featuring a 2.8” backlit LCD status screen, capacitive touch buttons, a tamperproof screen lock, and a lacquered PCB for enhanced protection against moisture and dust. The package also includes a 7m, RJ11 control cable, door contact and a handheld infrared remote control.

- The controller features an integrated temperature sensor, with semi-automatic ECO mode for setpoint regulation, with manual override for temperature and fan speed adjustment. ECO Mode settings can be configured based on door status, including door delay (0 to 95 sec), maximum speed and heating (open and closed), maintenance/filter intervals (0 to 92 weeks), temperature boost mode, and additional related functions.

- The controller features ‘Plug & Play’, auto configuration with a backlit LCD showing temperature, fan speed, heating, door status, alarm/error and unit off. It also includes three digital inputs for BMS/timer enable/disable, heat enable/disable, fire alarm disable, with manual safety reset.

- The optional ‘Clever Pro’ controller includes all ‘Advanced Pro’ features (excluding fire alarm input), plus a 7-day timer, and Modbus RTU compatibility with BMS, for remote control.

- The Dam air curtain is produced to ISO 9001 standards, fully run tested for reliability, with traceable documentation for quality, is CE & UKCA certified and compliant with relevant regulations. BIM files can be provided upon request.

- The system is to be supplied by					www:	Dam Air Curtain

JS Air Curtains Limited

City Gates, 2-4 Southgate

Chichester

West Sussex 							Tel:	+44 (0)333 038 4240

PO19 8DJ	 							e-mail:	sales@jsaircurtains.com

United Kingdom							Website:	www.jsaircurtains.com