# JS Air Curtains Essensse Neo Air Curtain Specification

- The Essensse Neo Air Curtain features a low profile design for horizontal installation, with a self-supporting galvanised steel casing. The casing has rounded edges giving it an aesthetic look.

- The Essensse Neo casing is finished with a structural epoxy-polyester paint finish, offered in standard RAL 9016 (White), with other RAL classic colours available on request.

- The unit is equipped with unique Straw System Technology to maximise the screening effect. The Straw System ensures a laminar, compact and stable airflow across the entire exhaust nozzle.

- As standard, the fans are ‘AC’ type with tangential design, featuring low noise levels. They use an external rotor motor, offer three selectable speeds, and come equipped with built-in thermal protection.

- EC (electronically commutated) fans with low energy requirements can be specially ordered, offering enhanced energy efficiency, lower operating costs, and sophisticated control options.

- The Essensse Neo unit shall measure 252mm high by 407mm deep and can be supplied in various lengths ranging from 1.0m to 2.5m in 0.5m increments.

- The unit is designed for exposed, horizontal mounting above doors utilising the supplied wall mounting brackets. Alternatively, M6 threaded drop rods can be used.

- The air inlet grille is located in the upper part of the unit making it an attractive air curtain for commercial entrances. Only routine cleaning or vacuuming is needed in normal conditions.

- The unit can be specified with two regulated heating options: ‘E’ type (electric element heating with three stages), or ‘P’ type (LPHW water coil).

- E’ type electric units shall operate on 400V/50Hz three-phase power and include solid state relays with ten selectable heating stages.

- ‘P’ type water heated units shall offer on/off or proportional heating control. Automatic external solenoid valves shall also be available on request.

- The Essensse Neo is equipped with an ‘AirGenio Prime’ controller, featuring an intuitive touch screen display, built-in temperature sensor, 7-day timer, volt free contact, and automatic control with manual override. The AirGenio application offers advanced settings and remote control functions.

- The AirGenio Prime system supports master and slave operation, with one master unit capable of controlling up to 10 slave units. Each master unit shall be supplied with 20m of control cable.

- Optional AirGenio Prime gateways shall be available for enhanced connectivity. BMS Gateway shall enable integration using Modbus TCP or BACnet protocols. Cloud Gateway shall allow Prime-controlled devices to be monitored, controlled and managed remotely through a web interface from any location.

- The Essensse Neo air curtain is produced to ISO 9001 standards, fully run tested for reliability, with traceable documentation for quality, is CE & UKCA certified and compliant with relevant regulations. BIM files can be provided upon request.

- The system is to be supplied by					www:	Essensse Neo Air Curtain

JS Air Curtains Limited

City Gates, 2-4 Southgate

Chichester

West Sussex 							Tel:	+44 (0)333 038 4240

PO19 8DJ	 							e-mail:	sales@jsaircurtains.com

United Kingdom							Website:	www.jsaircurtains.com