# JS Air Curtains Fly, Air Curtain Specification

- The Fly Air Curtain unit is specifically designed for insect control at entrances to food factories, industrial applications, hospitals, or clean zones, and has a self-supporting galvanised steel casing.

- The Fly casing is finished with a structural epoxy-polyester paint finish, offered in standard RAL 9016 (White), with other RAL classic colours, or stainless steel, available on request.

- The unit operates using AC or EC (electronically commutated) double-inlet centrifugal fans (depending on the application) driven by an external rotor motor and featuring low noise levels. With low energy requirements, the EC fans offer enhanced energy efficiency, lower operating costs, and sophisticated control options.

- The Fly unit is available in four models - Fly K, Fly KBB, Fly KL and Fly KXL - with varying dimensions and mounting heights. These can be supplied in lengths ranging from 1.0m to 3.0m in 0.5m increments.

- The units are designed for exposed mounting above doors utilising M8 or M10 threaded drop rods. Optional brackets may be specially ordered for overhead or wall mounting, including, wall ‘rail’ supports, suspension cables.

- The air inlet grille is perforated, with filter function, to allow easy servicing, so only routine cleaning or vacuuming is needed in normal conditions.

- The unit is specified as ‘A’ type only (low-energy air, unheated or recirculating ambient air).

- The unit comes with anodised aluminium vanes shaped like air foils to minimise drag and noise. Each vane can be adjusted from 0 to 15º from the vertical to control the direction of airflow.

- The Fly is equipped as standard with an ‘Advanced Pro’ digital controller, featuring a 2.8” backlit LCD status screen, capacitive touch buttons, a tamperproof screen lock and a lacquered PCB for enhanced protection against moisture and dust. The package also includes a 7m, RJ11 control cable, door contact and a handheld infrared remote control.

- The Advanced Pro controller features fan speed adjustment based on door status, including door delay (0 to 95 sec), maximum speed for door state (open and closed), and maintenance/filter intervals (0 to 92 weeks).

- The controller features ‘Plug & Play’, auto configuration with a backlit LCD showing fan speed, door status, alarm/error and unit off. It also includes two digital inputs for BMS/timer enable/disable, fire alarm disable, with manual safety reset.

- The optional ‘Clever Pro’ controller includes ‘Advanced Pro’ features (excluding fire alarm input), plus a 7-day timer, and Modbus RTU compatibility with BMS, for remote control.

- The Fly air curtain is produced to ISO 9001 standards, fully run tested for reliability, with traceable documentation for quality, CE & UKCA certified and compliant with relevant regulations. BIM files can be provided upon request.

- The system is to be supplied by:

JS Air Curtains Limited 						www:	Fly Air Curtain

City Gates, 2-4 Southgate

Chichester

West Sussex								Tel:	+44 (0)333 038 4240

PO19 8DJ 								e-mail:	sales@jsaircurtains.com

United Kingdom 							Website:	www.jsaircurtains.com