# JS Air Curtains VCP Air Curtain Specification

- The VCP is a heavy-duty industrial air curtain designed for vertical or horizontal installation, with self-supporting galvanised steel casing.

- The VCP casing has a galvanised finish with the option of a RAL 9016 (White) paint finish.

- The unit is equipped with unique Straw System Technology to maximise the screening effect. The Straw System ensures a laminar, compact and stable airflow across the entire exhaust nozzle.

- As standard, the VCP fans are ‘AC’ type. Low noise axial fans are driven by a 230V single phase motor, offer five selectable speeds, and come equipped with built-in thermal protection.

- EC (electronically commutated) fans with low energy requirements can be specially ordered, offering enhanced energy efficiency, lower operating costs, and sophisticated control options.

- The VCP unit shall measure 521mm high by 551mm deep and can be supplied in lengths of 1.65m, 2.20m and 2.75m.

- The unit is suitable for exposed, horizontal mounting above door, hung using M8 threaded drop rods, or vertical mounting to the side of the doorway using a base plate with floor fixing points. Optional brackets may be specially ordered for overhead or wall mounting.

- For larger doorways, up to six VCP units can be mounted horizontally to cover a width of up to 16.5m, or 3 units can be stacked vertically using a specially designed joining kit to form a tower up to 6.60m high.

- The galvanised sheet casing and closed construction motors with self-lubricating ball bearings provide easy maintenance. Only routine cleaning or vacuuming is needed in normal conditions.

- The unit can be specified with various regulated heating options: ‘S’ type (low-energy air only, unheated or recirculating ambient air), ‘E’ type (electric element heating with three stages), or ‘V’ type (LPHW water coil).

- E’ type electric units shall operate on 400V/50Hz three-phase power and include power contactors, with two standard heating stages.

- ‘V’ type water heated units shall offer on/off heating control with optional valve.

- The VCP is equipped as standard with an ‘AirGenio IC Prime’ digital controller, featuring 5 fan speeds, integrated timer, built-in temperature sensor, automatic antifreeze protection, and Modbus RTU compatibility with BMS. The AirGenio application offers advanced settings and remote control functions.

- The AirGenio IC Prime system supports master and slave operation, with one master unit capable of controlling up to 6 slave units.

- The VCP air curtain is produced to ISO 9001 standards, fully run tested for reliability, with traceable documentation for quality, is CE & UKCA certified and compliant with relevant regulations. BIM files can be provided upon request.

- The system is to be supplied by					www:	VCP Air Curtain

JS Air Curtains Limited

City Gates, 2-4 Southgate

Chichester

West Sussex 							Tel:	+44 (0)333 038 4240

PO19 8DJ	 							e-mail:	sales@jsaircurtains.com

United Kingdom							Website:	www.jsaircurtains.com